PHD RESEARCH / EXPERIMENT REPORT

# RAVEN

SuperMimic-Inspired Dynamics Learning
Daily progress, experiments and research findings

Research session: 25–26 October 2026  |  NVIDIA Isaac Sim / Isaac Lab  |  Unitree G1 + physical tripod

| 2 × 7,501 120 Hz state–action recordings | 100% RGB-D actor detection, both runs | 66.7 ms Eight-step predictive horizon |
| --- | --- | --- |

# Executive summary

The session established a reproducible data-to-model workflow for the RAVEN humanoid robotic cinematography system: validated camera tracking, full-rate G1/tripod telemetry capture, offline differentiable dynamics-model training, and evaluation on a separate actor trajectory. The working perception, assisted-grasp and tracking baseline was preserved.

NEW RESEARCH FINDING On the independently recorded trajectory, the learned world model reduced 66.7 ms pan-prediction error relative to assuming no motion (0.1933° versus 0.2433° MAE), but was less accurate than constant-velocity extrapolation (0.1677° MAE). This is evidence of useful learned motion prediction, not yet of superiority over a simple physics-informed baseline.

Figure 1. Independent trajectory: eight-step pan prediction MAE. The constant-velocity method is the strongest of the three tested predictors.

Research significance. The result motivates a residual dynamics model that learns deviations from constant-velocity motion, rather than modelling full pan dynamics from scratch. No reinforcement-learning policy has been trained or deployed.

# 1  Work completed during the session

IMPLEMENTATION AND VERIFICATION

| Stage | Delivered / observed |
| --- | --- |
| Repository and environment | GitHub main at 1e2ea38; working feature branch feature/supermimic-integration; PyTorch 2.7.0+cu128 and CUDA available on RTX 4060 Laptop GPU. |
| Full-rate capture | New raven_g1_rl_data_capture.py records 51 G1 joints, joint targets, measured velocities, tripod pan/tilt and RGB-D observation metadata at 120 Hz. |
| First recording | 7,501 state–action samples, 351/351 successful actor detections, original tracking run complete. |
| World-model V1 | PyTorch residual MLP; eight-step differentiable rollout; 60 training epochs; checkpoint and metrics exported. |
| Independent recording | Second 7,501-sample capture on a different actor motion trajectory, with complete camera tracking. |
| Out-of-sample evaluation | Frozen model tested on 1,049 overlapping eight-step windows; compared against stationary and constant-velocity baselines. |

# 2  Recorded experiments

| Capture characteristic | Recording A · training | Recording B · independent |
| --- | --- | --- |
| Run identifier | 20261025_230816_212515 | 20261026_004925_836186 |
| Physics samples | 7,501 | 7,501 |
| Physics rate | 120 Hz | 120 Hz |
| RGB-D detections | 351 / 351 | 351 / 351 |
| Actor motion amplitude | ±0.40 m | ±0.30 m |
| Actor motion period | 11 s | 7 s |
| Physical pan range | −8.69° to +9.25° | −6.56° to +6.80° |
| Tracking summary MAE | 0.349° | 0.410° |

Figure 2. Ground-truth actor lateral motion (used here only for experiment characterisation, not as controller input). The independent run is faster and has smaller displacement.

# 3  Differentiable world-model V1

A SuperMimic-inspired offline model was trained in PyTorch from state transitions recorded with Isaac Sim at 120 Hz. The architecture is a residual multilayer perceptron: it predicts a normalised next-state correction conditioned on the current state and the commanded joint-position targets. Its predictions are recursively unrolled for eight physics steps.

| Component | Configuration |
| --- | --- |
| State (20 values) | 8 selected G1 joint positions, 8 joint velocities, tripod pan/tilt positions and velocities. |
| Action (8 values) | Position targets for waist yaw and 7 left-arm joints; not direct torque commands. |
| Model | Residual MLP, 2 hidden layers of 128 units, SiLU activation. |
| Training | Adam; 60 epochs; 8-step rollouts; 120 Hz samples; CUDA on RTX 4060. |
| Single-run split | Chronological 70% train / 15% validation / 15% test, with rollout windows confined to each partition. |
| Original test windows | 734 training / 156 validation / 156 test. |

Figure 3. Logged training and validation normalised rollout MSE (selected epochs). Validation improves from ~0.472 at epoch 1 to ~0.158 at epoch 60.

## Initial within-trajectory evaluation

| 0.1521° World-model 8-step pan MAE | 0.2063° Stationary baseline MAE | 26.3% Relative MAE reduction |
| --- | --- | --- |

The initial result is promising but limited: the test windows were held out chronologically from the same simulated trajectory, which does not demonstrate generalisation to new operating conditions. This motivated the subsequent fully separate trajectory evaluation.

# 4  Independent trajectory generalisation

The trained checkpoint was frozen and evaluated on the second recording. The actor movement differed in both amplitude (±0.30 m) and period (7 s) from the first run. No model weights were updated using this second dataset.

STEP-8 PREDICTION: 66.7 MILLISECONDS AHEAD

| Method | Pan MAE | Pan P95 | Pan max error |
| --- | --- | --- | --- |
| Constant velocity | 0.1677° | 0.3387° | 0.4945° |
| Learned model | 0.1933° | 0.4810° | 1.1399° |
| Stationary (persistence) | 0.2433° | 0.4258° | 0.4874° |

For pan, the learned model is 20.55% better than stationary persistence, but its MAE is 15.26% higher than constant velocity. The learned model also has the largest observed tail error: 1.1399° maximum versus 0.4945° for constant velocity.

Figure 4. Independent pan-error P95. The learned predictor has poorer tail accuracy than both baselines despite improving mean error over persistence.

## Tilt predictions and capture integrity

| Tilt: step-8 MAE | Learned model | Stationary | Constant velocity |
| --- | --- | --- | --- |
| Degrees | 0.0225° | 0.0241° | 0.0913° |

For tilt, the learned model slightly improves on persistence and clearly beats a constant-velocity extrapolation. This indicates that pan and tilt have different predictive structure in the present constrained-tripod configuration. The independent run supplied 4,202 tracking-phase states and 1,049 overlapping evaluation windows, with no camera detection failures.

# 5  System-level evidence and setbacks

Figure 5. Vision-derived desired pan and physically observed tripod pan on the independent run. This graph shows the existing tracking controller, not world-model-driven control.

## Grasp ergonomics: open engineering issue

A side-view inspection revealed an unnatural left-elbow configuration near its upper flexion limit. A 10 cm closer operator stance was explored, together with faster grasp and manual calibration interfaces. The original tracking setup remained the reference baseline after these experimental variants did not produce reliable alignment.

| Experimental change | Observed issue |
| --- | --- |
| Closer operator + downward elbow objective | Pre-release pan drift reached +7.49°, beyond the ±0.75° permitted limit. |
| Closer stance with elbow-prep disabled | Pre-release pan drift of −1.42°; experiment stopped. |
| Closed-grip retargeting | Hand-to-target position error 87.2 mm; assisted grasp correctly refused. |
| Viewport gizmo calibration | Isaac Sim exited; provided shutdown warnings were insufficient to isolate the root exception. |

Decision for this research milestone Stop further calibration experiments temporarily. Preserve the successful original G1–tripod controller and progress with offline world-model validation; return to grasp ergonomics as a separately controlled engineering workstream.

## 6  Next research milestones

World Model V2: predict a learned correction to constant-velocity pan and an appropriate separate tilt prior, with regularisation against excessive overshoot.

Training diversity: train using multiple distinct trajectories and reserve a third, unseen motion profile for genuine episode-level evaluation.

Latency-aware evaluation: explicitly align sensor timestamps with robot/actuator states and measure performance under delayed or intermittent RGB-D observations.

Only after robust generalisation: evaluate model-based action optimisation offline, then integrate a guarded predictive controller into Isaac Sim without replacing the validated baseline.

# 7  Research interpretation and reproducibility

## Primary finding

In a camera-guidance task combining humanoid joint actuation, assisted grasp and a passive physical tripod, a learned eight-step world model provides useful short-horizon pan estimates compared with a motionless-state assumption. However, a low-complexity constant-velocity model generalises more accurately to the independently recorded, faster actor motion. Therefore, architectural inductive bias and appropriate motion priors may matter more than model capacity at this stage.

## Evidence boundaries

One additional trajectory establishes a limited cross-trajectory test, not broad statistical generalisation or a formal controlled benchmark.

The 1,049 evaluation windows overlap substantially and should not be interpreted as 1,049 independent episodes.

The predictor receives future recorded joint-position targets for each eight-step rollout; it has not yet learned to select actions, operate under unseen action sequences, or control the robot online.

The pipeline implements a SuperMimic-inspired differentiable dynamics model. It is not the published SuperMimic training system, and no reinforcement-learning policy has been optimised.

Perception logging records observation read-time age, not validated camera exposure timestamps; latency claims require dedicated sensor-frame synchronisation experiments.

## Source artefacts used

Primary experimental files (repository-relative paths):

scripts/raven_g1_rl_data_capture.py  —  120 Hz robot/tripod state–action recorder.

scripts/raven_supermimic_120hz_world_model.py  —  Offline residual world-model training implementation.

scripts/raven_supermimic_independent_eval.py  —  Frozen-checkpoint independent generalisation evaluator.

raven_training_results/supermimic_capture/20261025_230816_212515/  —  Recording A: original capture and world-model checkpoint.

raven_training_results/supermimic_capture/20261026_004925_836186/  —  Recording B: independent capture and evaluation metrics.

## Numerical definitions

MAE = mean absolute error; P95 = 95th percentile absolute error; RGB-D = colour plus depth imaging; IK = inverse kinematics; world model = learned mapping from current physical state and action to predicted future state. Smaller error is better for all quoted prediction metrics.

Recommended next experiment Implement and evaluate a physics-residual World Model V2, benchmarking against constant-velocity extrapolation on a third withheld actor trajectory. Advance to policy optimisation only if it improves both mean and tail prediction accuracy under unseen conditions.